ELEC2100				Labsheet #2

| 2 | 0 | 7 | 6 | 1 | 3 | 2 | 5 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Name	:		XUE Hongjia			Student ID :

Objective

- To be familiar with impulse response and convolution		Deadline : 19 Oct (20:00)

Part I		(7)

Let the value of A = the 8th digit of your student ID

Let the input sequence x[n] to be the same as your student ID.

e.g. Student ID = 20345678			xn=[2, 0, 3, 4, 5, 6, 7, 8]		for n = A to A+7

The impulse responses of two LTI systems are shown below.

System 1 :  h1n=δn-2+δ[n-4]		System 2 : h2n=un-A-3-u[n-A-7]

Assume that System 1 and System 2 are connected in series to give the overall system h[n].

The input sequence x[n] is applied to the overall system to give the output y[n].

a) Write down the mathematical expression of the overall impulse response h[n] in terms of h1[n] and h2[n].

(1)	hn= h1[n] * h2[n] ( * means convolution )

b) Define h1 and h2.

c) Use appropriate Matlab command to obtain the overall impulse response h according to the answer provided in (a).

d) Define nh index.

e) Fill in the following table after running your Matlab code.

(1)	Only show non-zero value of h and corresponding nh

| nh | 10 | 11 | 12 | 13 | 14 | 15 | 16 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| h | 1 | 0 | 1 | 0 | 1 | 0 | 1 |  |  |  |  |  |

f) Use appropriate Matlab command to obtain the output y and define the corresponding index ny.

g) Fill in the following table after running your Matlab code.

(1)	Only show non-zero value of y and corresponding ny

| ny | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 | 18 | 19 | 20 | 21 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 2 | 0 | 9 | 6 | 10 | 9 | 12 | 14 | 10 | 14 | 3 | 8 |

| ny | 22 | 23 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| y | 2 | 5 |  |  |  |  |  |  |  |  |  |  |

h) Plot the input in BLUE, the overall impulse response in RED and the output in GREEN using “subplot” and “stem”.

Label the axes. Place the screenshot of your simulation results in the space provided below.

(2)	figure(1)

(2)	Screenshot of Matlab code for Part I

Part II		(7)

Use “audioread” to read the audio file (song2c.wav) downloaded on Canvas.

a) What is the sampling frequency (fs) in Hz ?

(1)	The sampling frequency =  132300

b) What is the time interval (in seconds) between two consecutive sample points ?

(1)	The time interval = 1/132300 = 0.00000756 (s)

The impulse responses of two CT LTI systems are given below.

System 1 : h1t=δt 				System 2 : h2t=0.8 δ(t-0.35)

If these two CT LTI systems are converted into DT representation using the sampling frequency (fs),

c) How many zeros are needed to represent a time delay of 0.35 seconds ?

(1)	Number of zeros = 0.35*132300 = 46305

d) Write down the DT representations of System 1 and System 2.

(1)	h1n=	[1]				h2n= [  zeros(1,46305), 0.8 ]

If these two DT LTI systems are connected in parallel and added together to give the overall system,

e) Write down the mathematical expression of the overall impulse response h[n] in terms of h1[n] and h2[n].

(1)	hn= h1 + h2 = [ 1, zeros( 1, 46304 ), 0.8 ]

f) Define the overall DT impulse response h according to the answer provided in (e).

g) Apply the audio file to the overall DT system h to obtain the output y using “conv”.

h) Hear the original audio file and the output y using “sound”.

i) What is the sound effect introduced to the original audio file?

(1)	there is an echo with decreased amplitude in addition to the original sound.

(1) 	Screenshot of Matlab code for Part II

Part III		(6)

a) Use “imread” to read the image file (image2c.jpg) downloaded on Canvas.

b) Display the image file using “imshow”. Place the screenshot in the space provided below.

(1)	figure(2)

c) Apply the image file to a 2-D filter h1n=-3         -3         -3-3             24         -3-3         -3        -3 to obtain the output using “convn”.

d) Display the new image using “uint8” and “imshow”. Place the screenshot in the space provided below.

(1)	figure(3)

e) What does the 2-D filter h1n do to the image ?

(1)	sharpen the image to get its shape/contour

The impulse response of a 1-D filter is given below.

hn=0.05 δn+0.15 δn-1+0.2 δn-2+0.2 δn-3+0.2 δn-4+0.15 δn-5+0.05 δn-6

f) Use the above 1-D filter and proper matrix operation to define a 7-by-7 2-D symmetric filter h2.

(i.e. Transpose of h2 is equal to h2)

g) Apply the image file to h2 to obtain the output using “convn”.

h) Display the new image using “uint8” and “imshow”. Place the screenshot in the space provided below.

(1)	figure(4)

i) What does the 7-by-7 2-D symmetric filter h2 do to the image ?

(1)	it makes the picture blurrer

(1) 	Screenshot of Matlab code for Part III